### Handbook for the MGH-NMR Standard Processing Stream

11/10/2000

### Douglas N. Greve, Ph.D.

NMR Center, Massachusetts General Hospital

greve@nmr.mgh.harvard.edu

### INTRODUCTION

This document describes how to use the functional part of the Standard MRI Processing Stream for the MGH-NMR Center.  This functional MRI processing stream is referred to as the FreeSurfer Functional Analysis STream, or FS-FAST.  FS-FAST allows users to create statistical  maps of the functional data for overlay on a structural substrate. The structural portion of the standard stream is referred to as "FreeSurfer."  The "Standard Stream" refers to a set of imaging and naming conventions that, when followed, will allow the user to quickly and efficiently process functional and structural MRI data.  Features of FS-FAST include blocked analysis, event-related design and analysis, sub-TR estimation of the hemodynamic response, single-subject analysis, and group analysis.  Interfacing with FreeSurfer allows for automatic talairaching as well as a host of analysis techniques based on the reconstruction of the cortical surface (eg, morphometry, spherical morphing and inter-subject averaging, and cortical inflation and flattening) based on Dale, et al, 1999, and Fischl, et al, 1999.  The statistical maps can be viewed on same-slice anatomicals, high-resolution anatomicals, Talairach anatomicals, and on the cortical surface (folded, inflated, or flattened).

# NEW for 11/10/2000

- Pre-11/10/2000 studies need to run fixseqinfo-sess (see Appendix C).Motion Correction reports motion in physical units instead of voxels.inorm-sess creates a data integrity report file.paint-sess paints all post-stimulus delays.It is no longer necessary to create mosaics.Functional runs can be excluded using the Run-List File.BUG REPORTS, QUESTIONS, AND COMMENTS – READ THIS!Limited support is offered for FS-FAST by sending email to analysis-bugs@nmr.mgh.harvard.edu.  In general, you will need to supply enough information so that the problem can be recreated. As a minimum, include the following:Version Date of the Handbook you are using (eg, 8/24/00).Specify which machine/operating system you were on when the problem occurred. Specify which program (along with command-line options) you were running when the problem occurred.  NOTE: "program" means a program from the FAST package, not one that you have created that calls FAST programs.Specify the directory you were in when you ran the program.Include any text that the program printed out to the screen.Include the log file (if it exists).CONCEPTUAL FRAMEWORKOne of the goals of the FAST design has been to minimize and conceptualize the user input as much as possible. To this end, a framework consisting of six components has been specified:Study

- Specify subjects and the groups they belong to

- Analysis – within session averaging

- Define event types with paradigm file

- Define parameters for averaging (most for event related data)

- Resampling (spatial normalization)

- Talairach or sphere or "native space" (not resampled)

- Group definition

- Contrast: Define conditions/time-points to be compared.

- Visualization

- Slice based - 2D view (sliceview, xds)

- Volume based - 2D views in 3 cardinal planes (contains 3D information)

- Surface based

### DIRECTORY STRUCTURE AND NAMING CONVENTIONS: THE SESSIONS FORMAT

- This section describes how the data from a given session must be organized (ie, file and directory naming conventions) in order to use the commands in the following sections.  This structure will be referred to as the "Sessions Format". While there is some flexibility in the Sessions Format as to the exact naming of  files and directories, all of the sessions to be processed in the study must have the same exact naming convention.  If the data are unpacked and processed as described in this document, then the output will always conform to the Sessions Format. An example of the way the raw data is organized is shown below.  The SessionId is the directory name that identifies the session.  Below this, are several directories.  The functional runs are stored under the "bold" directory, each run with a separate directory.  The run directory, is a three-digit, zero-padded number.  They need not be contiguous.  Within each run directory is a functional volume stored in bfile format (ie, bshorts).  The stem is always "f".  The other directories have similar conventions (except 3danat which stores the data in COR format).

SessionId

bold

3danat

t1epi

t2epi

t2conv

002

005

006

seq.info

- Each stage of processing will add new files/directories to this structure.

- Note: the format of the seq.info file has changed as of 11/10/2000. Data unpacked after this date will have the new format. Data unpacked before this date can be converted using fixseqinfo-sess.

- Seq.info file: the seq.info file appears in each directory under the SessionId. This file provides information about the acquisition like the voxel sizes and TR. The seq.info file has a particular format.  A typical seq.info file is shown below:

- sequencename     ep2d_fid_ts_20b2604

- nrows            64

- ncols            64

- nslcs            16

- rowpixelsize     3.125

- colpixelsize     3.125

- slcpixelsize     5.000

- ntrs             100

- TR               2.0

- Seqencename is the name of the sequence and is only informational at this time. Nrows is the number of rows in each slice. Ncols is the number of columns in each slice. Nscls is the number of slices in the volume. Rowpixelsize is the physical width of a pixel in the  row dimension (in mm). Colpixelsize is the physical width of pixel in the column dimension (usually the same as that of the row). Ntrs is the number of TRs or frames in a functional volume. TR is the time between volume acquisitions (in seconds). For non-functional volumes, ntrs and TR can be omitted.

- Variation 1: The Functional Subdirectory.:  The actual name of the data directories may be different depending upon the sequence used to collect the data.  The directory in which the functional data reside is referred to as the functional subdirectory. If the functional subdirectory is not named "bold", it can be changed to "bold".  If it is something other than "bold", then the user must specify the name during processing using the –fsd flag. The functional subdirectory only needs to be specified during motion correction, intensity normalization, and analysis creation.

- Variation 2: Multiple Functional Subdirectories. Sometimes an experimenter will collect functional data under several different paradigms or slice prescriptions.  In these cases, all of the functional runs will initially reside in the bold directory.  If left in this state, they would all be processed as if they represented the same experimental protocol.  To get around this, the user should create a new functional subdirectory (say, "bold2") and move (not copy) the appropriate runs to the new directory.  The "seq.info" file should also be copied into the new bold directory (make sure that the seq.info file has the appropriate parameters).  In subsequent processing, the –fsd flag can be used to specify the functional subdirectory. Example: "-fsd  bold2".  The functional subdirectory only needs to be specified during motion correction, intensity normalization, and analysis creation.

- Variation 3: Excluding Runs. To exclude the data from a particular run, change the name of the run directory.  The software interprets any three-digit, zero-padded directory to contain functional data, so changing the name to something else will cause the software to ignore the run. For example, if run 5 had been aborted in the middle, its name could be changed from "005" to "005.abort". This will unambiguously exclude run 5 from any processing. One can also specify a run-list-file (see Step 10) which lists runs to include.  The run-list-file only affects the within-session analysis; runs not included in the run-list-file will still be motion-corrected and intensity normalized.

### Overview of Functional Processing Steps

- The standard data analysis stream includes

- Unpacking data from the scanner		(browse-sessions: Linux)

- Motion correction				(mc-sess:              Linux, SGI, Sun)

- Signal intensity normalization		(inorm-sess:         Linux, SGI, Sun: matlab )

- Analysis definition				(mkanalysis-sess: Linux, SGI, Sun: matlab)

- Within session averaging			(selxavg-sess:       Linux, SGI, Sun: matlab)

- Spatial normalization 				(func2tal-sess, sph2tal-sess: Linux, SGI)

- Inter-session averaging			(isxavg-fe-sess    Linux, SGI, Sun: matlab)

- Contrast definition				(mkcontrast-sess: Linux, SGI, Sun: matlab)

- Statistical map computation			(stxgrinder-sess:   Linux, SGI, Sun: matlab)

- Visualization

- Slice-based: (sliceview-sess, mkmosaic-sess: Linux, SGI, Sun: matlab)

- Surface-based:  (paint-sess, surf-sess: Linux, SGI)

- Before running any of these programs, you should source the NMR Standard Environment file:

- source ~inverse/freesurfer_alpha/nmr-std-env

- This will set up your environment properly (ie, set the appropriate variables and paths). Everyone in the MGH-NMR Center can log into either of two Linux computers: bonaire and ichiban.  If you rlogin to either of these, then you will probably need to set your display variable:

- setenv DISPLAY yourcomputername:0.0

- A feature of the standard data processing stream is that it allows you to analyze data for all subjects at once.  To do this, you will create a directory that will contain the analyzed data.  This directory may be separate from the directories containing the individual subjects' data.  You will also create two text files that identify the name and location of the individual subjects' data directories.  If you wish to analyze the data for a single subject, you can do this by either including information about only a single subject in these text files, or by identifying the single subject at the command line when running the analysis programs.

- You will need approximately 300-400 MB per subject, depending on number of functional runs.

- Note: The following instructions are for standard use of the analysis tools.  For special conditions, see the other command-line options detailed at the end of this document.

- 1. Unpack individual subject's data to any disk space

- After scanning, the data for the subject is saved onto the NMR Center repository.  The repository can only be accessed from certain computers.  At the time of this writing, there are two machines from which data can be retrieved: ichiban and bonaire.  The example below uses bonaire; to use ichiban, simply substitute "ichiban" for "bonaire".

- 1. On your computer, type:   xhost + bonaire

- If you get an error of the form "xhost: Command not found.", then

- (a) If you are on a Sun,  type: /usr/openwin/bin/xhost + bonaire

- (b) If you are on an SGI or  Linux, type: /usr/bin/X11/xhost + bonaire

- 2. ssh bonaire (if ssh fails, use ssh1; if ssh1 fails use rlogin)

- 3. source ~inverse/freesurfer_alpha/nmr-std-env

- 4. setenv DISPLAY yourcomputername.nmr.mgh.harvard.edu:0.0

- 5.  Type: browse-sessions

- This will bring up two windows, one called "Browse" and the other  called "Progress". The Browse window will have a list of sessions (ie, subject names and dates) as well as some buttons.

- 6. Select the session to unpack by clicking on it with the mouse. Only one session can be selected.  A search can be launched for a particular session or subject from the Options menu.

- 7. Select the destination directory by editing the text in the box labeled "Destination Directory" in the Browse window.  If the directory does not exist, it will be created.  By default, the Destination Directory is set to the directory in which you ran browse-sessions (which is probably not where you want the data to go).

- 8. Click the "start unpacking" button.

- This unpacks the functional data from .ima (Siemens) files into .bshort files (by first converting to MINC format). To create MINC files only, under OPTIONS, choose MINC only.)

- 9. Data is unpacked when message "COMPLETED SUCCESSFULLY" appears in the Progress window.

- 10. Kill the Browse window or select another session to unpack.

- Notice that several subdirectories and a file are created under the subject directory. The subdirectories correspond to the imaging data collected using the standard brain mapping protocol (3danat, bold, scout, t1epi, t2epi, t2conv).   In addition, a text file (session.info) is created containing some general information (DOB, gender, console name, study date).

- 2. Make a directory for your study.

- mkdir MYSTUDY (this will be referred to as the "Study Directory")

- cd MYSTUDY/

- 3. Specify the subjects to process

- The standard stream will process all of the subjects in a study. This is done by specifying a list of Destination Directories.  The Destination Directory for a subject was entered into the Browse window during the Unpacking stage.  To do this, each Destination Directory is broken into two parts:  the session identifier and the session directory.  For example, if the Destination Directory is "/space/someday/users/brainmapper/20001124DG", then the session identifier is "20001124DG" and the session directory is "/space/someday/users/brainmapper/".  The session identifiers should be entered into a text file (hereafter referred to as the "sessidfile"), and the session directories should be placed in a another text file (hereafter referred to as the "sessdirfile").  You can name these files anything you want. These names will be needed in further processing steps. If you have more than one group, you can make a separate sessidfile for each.  If the same session directory applies to more than one session identifier, then it need only appear once in the sessdirfile. Note that there need not be a one-to-one correspondence between the entries in the sessidfile and the sessdirfile.

- Example: if you have three subjects (DG, JG, and JM) in your study and their data were unpacked to /space/monday/users/000328DG, /space/tuesday/users/000329JG, and  /space/monday/users/000328JM,

- your sessidfile would look like this:

- 000328DG

- 000329JG

- 000328JM

- and your sessdirfile would look like this:

- /space/monday/users

- /space/tuesday/users

- When invoking commands in the standard processing stream, you will need to specify the subjects to process.  This is done primarily by adding several command-line arguments:

- -sf sessidfile1 –sf sessidfile2 …

- -df sessdirfile

- where "sessidfile1" is the session identifier file for one group, "sessidfile2" is that of a second  group (if necessary).  If there where a third group, then that would be indicated by adding a third "-sf" option.  Adding these options will instruct the program to process all of the subjects without any further input from the user.

- Variation:  Specifying individual sessions.  The processing stream is designed to analyze the data from all sessions at the same time; however, it is often the case that a user wants to process the data from a single session without having to reprocess the data for the other sessions (eg, after a session has just been collected).  In this case, the user has two options: create a new sessidfile in which only the name of the target session appears or to use the –s option instead of the –sf option.  The –s flag should be followed by the ID of the target session.  Example: "-s 000329JG".

- 4. Motion correct data with mc-sess

- This script uses the AFNI motion correction algorithm to align all the images in a session to the

- first image of the first run.  To view the time courses of raw or motion corrected data, see the variation for Step 20.

- To motion correct, cd to the Study Directory and type

- mc-sess –sf sessidfile –df sessdirfile

- If the functional subdirectory is not named "bold", then invoke mc-sess as

- mc-sess –sf sessidfile –df sessdirfile –fsd funcsubdir

- where "funcsubdir" is the name of the functional subdirectory.

- This command will create a new functional volume in each run subdirectory in the Sessions Format.  The new volume will have the same size as the raw volume but will have stem "fmc" instead of "f".  A text file called fmc.mcdat will also be created in each run subdirectory.  The motion correction parameters will be stored in this file.  Each row corresponds to a different time point.  The first 7 columns have the following interpretation: (1) time point number, (2) rotation about the row direction, (3) rotation about the column direction, (4) rotation about the slice direction, (5) translation in the row direction, (6) translation in the (negative) column direction, (7) translation in the slice direction.  The rotations are in degrees, and the translations are in mm. Column 8 is a measure of the mismatch before correction; column 9 is the measure after correction. Column 10 is the total (vector) translation in mm. This data file can be plotted using gnuplot or other plotter (or exported and plotted in e.g. Excel).  Note that this step is optional. Even if performed, the users will still have a choice later in the analysis stream as to whether to use raw or motion corrected data. One may want to motion correct in order to determine how much motion there was.

- Note: prior to 11/10/2000, the translations listed in fmc.mcdat were measured in voxels, and the rotations were not easily interpretable. After this date, the translations are in mm, and the rotations are in degrees. The motion correction itself should be unchanged.

- Note: for data unpacked before 11/10/00, the user must run fixseqinfo-sess before running this command.  See appendix C.

- 5. Normalize signal intensity

- This produces a file with mean intensity data for the entire functional volume for each raw and motion corrected run.  This step is optional but highly recommended when the data are to be averaged across subjects. It also produces a report for each run that can be used to asses the stability of the scanner during the session.

- To intensity normalize the raw data, cd to the Study Directory and type

- inorm-sess –sf sessidfile –df sessdirfile

- To intensity normalize the motion corrected data, cd to the Study Directory and type

- inorm-sess –sf sessidfile –df sessdirfile -motioncor

- If the functional subdirectory is not named "bold", then invoke inorm-sess with "–fsd funcsubdir" where "funcsubdir" is the name of the functional subdirectory.

- After running this command, you will see a file called "f.meanval" for the raw data or "fmc.meanval" for the motion corrected data in each run subdirectory. There will also be a report file called “f.report” as well as two files called “f.twf-under” and “f.twf-over”.  The report file has a host of statistics, the most important of which are: “OV ZMax” and “OU Mean”. The ZMax statistic is an indication of spiking or instability. The OU Mean statistic is a measure of distortion.  These are meant to be indicators of a possible problem during scanning; however, it is difficult to assign hard thresholds below which the user need not investigate the integrity of the data. The user should be examine these statistics for all the sessions in a study in order to determine appropriate levels. As a rule of thumb, a ZMax over 3.5 or OU Mean under 25 should trigger a closer look at that run. For backwards compatibility, inorm also reports the same statistics produced by an older program called “stackfix”.

- Note: these statistics in the report file are available as of 11/10/2000.

- Note: for data unpacked before 11/10/00, the user must run fixseqinfo-sess before running this command.  See appendix C.

- 6. Make paradigm file(s) for your experiment.

- The paradigm file simply describes what was going on during your study with respect to time of presentation and stimulus type.  You will create one paradigm file for each run and place a copy of the paradigm file into the directory with that run's .bshort files. The paradigm file is a text file containing two columns of numbers. The first column contains the stimulus onset time, in seconds, relative to the time at which the first stored image of the run was collected.. The second column represents the condition code for each stimulus.  By convention, the code zero represents the baseline or fixation condition. The other conditions are to be numbered sequentially starting with 1. Give the file a name that describes your study, e.g. motorstudy.para.  The .para extension is not required, but helps to identify the file as a paradigm file.

- For the event related paradigms, the time (column one) should be the time of stimulus onset and need not be an integer multiple of the TR.  For example, in the paradigm file below, the first stimulus (condition 1) was presented 2 seconds before the first image was acquired, the second stimulus (condition 3) at the first image, and the third stimulus (fixation) at 4.5 seconds into the imaging run.

- -2.0	1

- 0.0	3

- 4.5	0

- For block design paradigms there will be a line for each image. The first column is the TR onset. The second column is the condition code for the current block.  For example, if you have 2 12 second blocks and a TR of 3 seconds (starting with the baseline condition), the paradigm file will look like this:

- 0 	0

- 3 	0

- 6 	0

- 9 	0

- 12	1

- 15	1

- 18	1

- 21	1

- See Appendix B for how to perform event-related type of analysis on blocked designs.

- 7. Make Time-Point Exclude Files (if necessary) for each run.

- It is sometimes the case that one desires to exclude certain images or time points within an acquisition run.  This may be necessary if the scanner spikes during the run.  To do this, one creates a tpexclude file.  This file is similar to a paradigm file in that it lists the times (in seconds) for which an image should be excluded.  Unlike a paradigm file, there is only one column of numbers.  The tpexlude file should also go into the same directory as the raw functional data.

- 8. Set up Session-Level Analysis  using mkanalysis-sess

- With this program you define how the raw or pre-processed data for each session will be averaged together.  Data are not actually averaged at this stage. You may wish to analyze your study in more than one way. For example with and without motion correction, or you may wish to define your events base on subject response in one analysis and on stimulus type in another . You will give each analysis a name ("analysisname") that will be used in subsequent processing steps. For multiple analyses, you will run mkanalysis-sess once for each analysis, giving each a different analysis name.

- This program creates a subdirectory called "analysisname" in the study directory and writes several files there as well.  These include the analysis.cfg (see below) and analysis.info. It also checks that all subject data is accessible and creates default scripts for the analysis.

- When running mkanalysis-sess, you may include some of the following parameters:

- -analysis analysisname	name of analysis

- -paradigm parname		name of paradigm file

- -runlistfile filename		name of file with a list of runs to include (NEW to 11/10/00)

- -tpexclude filename		this filename refers to the time-point-exlude file described above

- -blocked			use for block design study

- -motioncor			use motion corrected rather than raw data

- -inorm				use intensity normalized data

- -sf 	sessidfile		see above

- -df	sessdirfile		see above

- (for more advanced customized analyses, see additional options in Appendix)

- NEW TO 11/10/00: The Run-List File.  At times it is desirable to process a subset of the runs in a particular session or to process different subsets in different ways.  To facilitate this, the user can create a Run-List File in the functional subdirectory with a list of the runs to include.  The name of this file is then passed with the “-runlistfile” flag to mkanalysis-sess. Like the paradigm file, there may be multiple run-list files but they must all be named the same across sessions but they need not have the same content.

- 9. Configure the Session-Level Averaging Parameters

- In the study directory, cd to analysisname

- Edit the file called analysis.cfg using any text editor.

This file contains information that is specific to your study that will be used in averaging data across conditions. Some of the parameters refer only to event-related designs, some only to block designs.

| Parameter | Description |
| --- | --- |
| -timewindow | Time needed (in seconds) to capture full hemodynamic response, including prestimulus baseline. |
| -prestim | Pre-stimulus window. Time in seconds to include in hemodynamic curve before stimulus onset. |
| -detrend | Remove linear trend and baseline (to shut off detrending, do not include, to just remove baseline, use -baseline option) |
| -baseline | Remove baseline (redundant with -detrend) |
| -nskip | Number of images at the beginning of each run to skip |
| -hanrad | Size of spatial smoothing filter (in in-plane pixels). (Any value less than one is equivalent to no filter.) |
| -rescale | Signal intensity value to which global mean will be normalized, based on values form inorm, above. |
| -gammafit | Parameters of gamma function for estimated hemodynamic response, for block design only. (First parameter is hemodynamic delay, second is dispersion, both parameters in seconds.) |

Setting the time window and pre-stimulus window.   The time window consists of a pre-stimulus-onset window and a post-stimulus-onset window.  The post-stimulus window should be set long enough that the hemodynamic response to an isolated stimulus presentation can be fully captured. I.e., the response will rise to some maximum and then decay back to baseline.  The amount of time needed depends upon the duration of the stimulus and the decay time.  While the decay time is not known in advance, one can comfortably assume that it will be less than 14 seconds.  Thus, the rule-of-thumb is to set the post-stimulus window to be the maximum stimulus duration plus 14 seconds.  The purpose of the pre-stimulus window is to assure that there is no activity before then onset of the stimulus.  Pre-stimulus activity is an indication that (1) the data are bad, (2) the paradigm file is wrong, or (3) your subject is psychic.  As a rule-of-thumb, use 2 TRs for the pre-stimulus window.  There is a penalty to be paid for lengthening the time window in that it reduces the degrees-of-freedom.

10. Average Session-Level Data by condition using selxavg-sess

This program computes the average signal intensity maps for each condition for each individual subject. The output is stored in a subdirectory called "analysisname" under the bold subdirectory in the subject's directory. (The "analysisname" was defined by the user using mkanalysis-sess, above). This average data can be further processed on an individual basis and/or can be used in group analysis.

In the study directory, run selxavg-sess with the following options:

-analysis analysisname	name of analysis

-sf 	sessidfile		see above

-df	sessdirfile		see above

This will create a directory called "analysisname" under the bold directory in each session.  There will be two volumes in this directory, one with stem "h" and one with stem "h-offset".

These are where the hemodynamic averages are stored.  Future processing of this analysis will create subdirectories under "analysisname".

11. Define a comparison to test using mkcontrast-sess

This program specifies which conditions will be compared. (The contrast is not actually computed at this step.) You will supply a name for this comparison which will be required at subsequent processing stages.  For multiple comparisons, you will run mkcontrast-sess multiple times, defining a different contrast name each time. The contrast is defined with respect to a particular analysis using the program options.

In the Study Directory, run mkcontrast-sess with the following options:

-analysis analysisname	name of analysis

-contrast contrastname	name of contrast

-a active condition code	to specify more than one active condition use multiple -a options

-c control_condition_code	to specify more than one active condition use multiple -c options

This will create a file called "contrastname.mat" under the "analysisname" directory in the Study Directory. Condition codes are same as used in paradigm file specified for this analysis in mkanalysis-sess, above.

Example 1:

For an analysis called "fingertapping", there are 2 conditions, condition 1 (right hand movement) and condition 0 (fixation).  The comparison will be called "righthand".  To compute this contrast use:

mkcontrast-sess -analysis fingertapping -contrast righthand -a 1 -c 0

Example 2:

For an analysis called "sentence-type", there are 8 conditions representing normal and anomalous sentence structure. The comparison will be called "ANOMvNORM".  To compute this contrast use:

mkcontrast-sess -analysis sentence-type -contrast ANOMvNORM -a 3 -a 4 -c 1 -c 2 -c 6

Note that there need not be equal numbers of active and control conditions and all condition types need not be represented.  For event-related paradigms, one can also specify delay contrasts in addition to condition contrasts.  Note also that the assignment of conditions to "control" and "active" is somewhat arbitrary in that if the designations are reversed, it only changes the sign of the final statistic, not its significance.

Customizations: for event-related designs it is possible to specify a "delay" contrast in which the users can specify which post-stimulus delay averages should be used in the computation of the statistic.  Also for event-related designs, it is possible to specify a family of ideal hemodynamic curves with which to correlate the averages.  This is done with the "-ircorr", "-deltarange", and "-taurange" flags.

12. Compute map of comparison using stxgrinder-sess – Individual

This program is used to create statistical, significance, and parametric maps for all of your subjects on an individual basis.  (Note that the same program with different options is used for the group analysis. See below.) The map is computed from a given analysis and contrast. Significance results are stored as log10 unless you specify otherwise using -pxform option.

In the Study Directory, run stxgrinder-sess with the following options:

-analysis analysisname	name of analysis (from mkanalysis-sess)

-contrast contrastname	name of contrast  (from mkcontrast-sess)

-sf 	sessidfile		see above

-df	sessdirfile		see above

This will create a subdirectory called "contrastname" under the bold/analysisname directory for each session.  Within this directory, there will be 4 new volumes with stems "sig", "minsig", "iminsig" and "t".  The variate-by-variate t-statistic is stored in the "t" volume and its significance in the "sig" volume. The variate is a time-point (or ideal hemodynamic curve). Multiple variates only apply to event-related design. In addition, the best variate (as measured by the minimum significance) at each voxel is also determined and stored as a map (after bonferroni correction) in the "minsig" volume.  The index of the best variate is stored in the "iminsig" volume.  In subsequent stages, you will be required to enter a map. This refers to one of these volumes.

Variation: Grinding individuals after reslicing.  After Steps 15-17, an individuals data will have been resliced into either talairach or spherical (or both)  space.  It is possible to compute and view statistics in these spaces by adding “-space tal” or “-space sph” to the command-line.

# 13. Anatomical Reconstruction

Further processing (eg, talairaching, cortical inflation) requires that the individual’s brain be anatomically “reconstructed” with the FreeSurfer software package. Reconstruction is the process of converting high-resolution T1 data (collected with the 3D_Flat_Sag_Opt_STD or 3D_Flat_Sag_STD scanning sequences) into a single volume/surface representation. Once reconstructed, the subject’s functional data can be viewed overlaid upon a 3D volume and/or inflated surface, talariached, and spherically morphed. It is only necessary for a subject to be reconstructed once.  After a subject has been reconstructed, then the experimenter only needs to collect “fast” anatomicals using the 3D_Quick_Sag_STD scanning sequence (see http://www.nmr.mgh.harvard.edu/Siemens_Scanner_Notes5.html for more information on the sequences that are needed for reconstruction).  The fast anatomical is used to register the session with the reconstructed volume.

Because reconstruction is a time-consuming process requiring much skill, the NMR’s Computational Core provides a reconstruction service.  A reconstruction can be requested by sending email to recon-request@nmr.mgh.harvard.edu. Requests are generally served on a first-come-first-serve basis. There is currently about a two-week turn-around for reconstructions. Include the following four pieces of information in your request:

- Location of the anatomical data to use in reconstruction.

- Acquisition information

- Contact information

- Subject Identifier String

(1) Location of the anatomical data to use in reconstruction.  If you have unpacked your data using browse-sessions (Step 1), then you only need to send the location of the session directory. Otherwise, you will need to send the full location of each file needed for the reconstruction.  Make sure that the permissions are set such that the data can be read by user inverse. (2) Acquisition information: type of scanner (GE/Siemens; 1.5T, 3.0T), scan sequence used (if not 3D_Flat_Sag_Opt_STD), institution where data was collected (eg, MGH-NMR).  (3) Contact information: full name, e-mail address, phone number.  Note: there should be only one contact person for each reconstruction request.  Collaborators should coordinate with each other as to who the contact person will be so that multiple requests to reconstruct the same data are not made.  (4) Subject Identifier String.  This is an alpha-numeric string of characters with which this individual will be associated.  It can be a full name (e.g. “john_smith”) or a code (e.g., “sh5694-lq5”).  Please be aware of any confidentiality issues when selecting your Subject Identifier String.  If the string you supply is already in use, you will be requested to supply another string.  If desired, the reconstructor can create a string for you. Create a file called “subjectname” in the SessionId directory, and put the Subject Identifier String (and nothing else) in this file.  Note that if the subject has been reconstructed from a previous session, he/she does not need to be reconstructed again.  Simply create the subjectname file in the new SessionId directory.

Upon submitting a request, you will receive an acknowledgement via email.  The reconstruction will be accomplished in two stages.  After the first stage is complete, you will be able to register your functional data with the anatomicals (Step 14), to resample your functional data into talairach space (Step 16), to view data in 3D with tkmedit-sess, and to view data on a preliminary cortical surface (Step 22).  After stage two, you will have final surfaces with which you can resample into spherical space (Step 17) and view your data on a flat-map. The first stage should be accomplished within 1 week at which point you will receive an e-mail notification. Along with this notification, you will receive the Subject Identifier String (see above).  After another week, you will receive e-mail notifying you that the second stage is finished.

# 14. Functional/Structural Registration

The functional volumes for each subject need to be registered with the anatomical volume in the Anatomical Database.  This is difficult to do because the two volumes have different resolutions and different contrasts and may have been collected on different sessions. To mitigate this problem, the registration will be accomplished in two stages: Automatic and Manual. The automatic is used to get the registration close, and the manual is used to fine-tune the registration.

Automatic Stage: the automatic registration can also be broken into two conceptual steps, though both are accomplished with one command.  The first step is the Header Registration in which the functional volumes are registered with the anatomicals that were collected during the same session.  No MRI data is used in the header registration.  It relies on the assumption that the head has not moved so that registration can be accomplished simply by registering the fields-of-view. In the second step, the same-session anatomical is registered with the Database anatomicals.  This is accomplished by translating/rotating the volumes based on a voxel-by-voxel comparison. This can be done because both volumes have T1 contrast.  The automatic registration is executed by running autoreg-sess from inside the Study Directory with the –sf and –df options.  This will create a file called register.dat in each functional subdirectory.  This represents the registration between the functional volume and the Database anatomical volume.

Note: for data unpacked before 11/10/00, the user must run fixseqinfo-sess before running autoreg-sess.  See appendix C.

Manual Stage:  because of movement and EPI distortion, the automatic stage can only get the registration close.  The manual stage allows the user to check and fine-tune the automatic registration. To run the manual registration, cd to the Study Directory and run tkregister-sess with the –sf and –df options.  This will read the register.dat created by the automatic registation and bring up two windows: an image window and a control window.  The image window will initially have a coronal view of  the anatomical in the Database.  Hitting the COMPARE button will switch views to the functional volume.  When the volumes are properly registered, the anatomical features in the functional slice should perfectly overlay those in the anatomical slice.  Hitting the SAGITTAL button will change the view to sagittal.  Hitting the HORIZONTAL button will change the view to axial.  The slider under each view button can be used to change the slice viewed.  The current slice can also be changed by clicking in the image window (moving the red cursor). With these operations, the user can navigate through the volume and check the registration.  If the registration is off, then the user has three options: translation, rotation, and scaling, each of which has it’s own frame inside  the control window.  Within the Translation frame, there are two sliders.  The vertical slider translates the functional brain in the vertical direction, where “vertical” is with respect to the current view in the image window.  The horizontal slider translates the functional brain horizontally.  The Rotation frame has only one slider.  Moving the slider will rotate the functional brain around the current cursor location about the axis perpendicular to the current image view.  The Scale frame has two sliders.  The vertical slider will stretch/compress the functional brain in the current vertical direction. The horizontal slider does the same for the current horizontal direction.  When finished, hit the SAVE REG button (it may prompt you to overwrite the current registration; if so, do so).  To quit, hit the button on the upper left hand corner to bring down a menu and choose the Close option. This will update the register.dat.  Note that re-running autoreg-sess will cause the results from the manual registration to be overwritten.  Also note that any changes in registration will require that all down-stream operations (eg, resampling, inter-subject averaging) be rerun.

Note: for data unpacked before 11/10/00, the user must run fixseqinfo-sess before running tkregister-sess.  See appendix C.

Variation 1: Manual registration of t2conv before registration of functionals. One of the reasons that it is difficult to register the functionals is because they are typically very low resolution.  One can, however, first manually register the T2 Conventionals which are co-planar to the functionals but have 2-4 times the in-plane resolution.  To do this, call tkregister-sess with “-rsd t2conv”. This will create a register.dat in the t2conv directory, and the registration matrix inside register.dat will be copied to the register.dat in the bold directory.  Do not attempt to perfectly register the t2conv volume, just use it to get the registration close.  After registering with t2conv, re-run tkregister-sess without the “-rsd t2conv” to perform the final functional registrataion.

Tips for Manual Registration: Manual registration can be a tedious process.  It is recommended that the user first translate, then rotate, and, as a last resort, scale.  Keep in mind that a registration improvement in one part of the volume can degrade that in another part. It is best to make small adjustments at several locations around the brain, slowly converging on the optimum registration.

15. Spatial Normalization (also known as Resampling and Reslicing)

Spatial normalization is the process through which two or more subjects' brains are aligned or registered based on anatomical information.  This is so that when the subjects' functional data  are averaged together on a voxel-by-voxel basis, the voxels represent the same anatomical location in each subject.  The Talairach transform is the traditional spatial normalization technique. However, the spherical transform (for cortical surfaces) is also supported in the standard stream.  Henceforth, the term space will be used to refer to the way in which the functional data has been spatially normalized (or resampled).  There are three spaces that are supported in the standard stream: native (no resampling), Talairach, and spherical.  Resampling is always done on data averaged in an analysis (ie, not the raw data).

# 16. Talairach Resampling using func2tal-sess

Within the Study Directory, run func2tal-sess with the following options:

-analysis analysisname	name of analysis to resample

-res      resliceresolution         size of the pixel in talairach space in mm (1, 2, 4, 8)

-sf 	sessidfile		see above

-df	sessdirfile		see above

This will create a subdirectory called "tal" under the bold/analysisname directory for each session.  Within this directory, two new volumes will be created with stems "h" and "h-offset". The dimensions of this volume will depend upon the resolution specified.  Typically one uses 2 or 4 mm.  The smaller the resolution, the larger the volume and the more disk space and processing time required.  As a rule of thumb, the Talairach resolution should be about the same that of the underlying functional data.

# 17. Spherical Resampling using func2sph-sess

Within the Study Directory, run func2sph-sess with the following options:

-analysis analysisname	name of analysis to resample

-sf 	sessidfile		see above

-df	sessdirfile		see above

This will create a subdirectory called "sph" under the bold/analysisname directory for each session.  Within this directory, two new volumes will be created with stems "h" and "h-offset".

18. Fixed-effects Inter-subject Averaging using isxavg-fe-sess

Once the subjects have been resampled into a common space (Talairach or spherical), the averages from the individual subjects can be averaged together using a Fixed-Effects model (see Step 25 for Random Effects averaging).  You may want to combine all of your subjects into a grand average, or you may want to divide your subjects into groups and compute grand averages for each group.  In any event, you will need to specify which subjects will be included in the average.  This collection of subjects will be referred to as a group, and you will be required to assign each group a name (groupname) which will be needed for subsequent processing.  Within the Study Directory, run isxavg-fe-sess with the following options:

-analysis analysisname	name of analysis to resample

-group groupname                  name of collection of subjects

-space  spacename                  name of the resampling space (tal or sph)

-sf 	sessidfile		see above

-df	sessdirfile		see above

This will create a directory called groupname in the study directory.  This directory will have the same structure as an individual session and can be accessed with the –s and –sf  flags as if it were an individual session.  Inside this directory, you will see bold/analysisname/spacename-ffx in which there will be two volumes, h and h-offset, in which the intersubject averages are stored.

19. Compute statistical map of comparison using stxgrinder-sess – Group

This program is used to create statistical, significance, and parametric maps for the grand average (fixed-effects) based on a particular contrast and analysis. Note that the same program with different options is used for the individual analysis. See that section for more detail.

In the Study Directory, run stxgrinder-sess with the following options:

-analysis analysisname   name of analysis (from mkanalysis-sess)

-s groupname                 name of collection of subjects in the grand average (from isxavg-fe-sess)

-space  spacename          name of the resampling space (tal or sph)

-contrast contrastname   name of contrast  (from mkcontrast-sess)

-sf 	sessidfile	   see above

-df	sessdirfile	   see above

This will create a directory called contrastname under groupname/bold/analysisname/spacename-ffx in the Study Directory.  See Step 12 for the list of output volumes created.

20. Visualization – Individual – Slice-based

In Slice-based visualization, the results are viewed on 2D slices of the brain (either individual slices or mosaics of slices).  No information about the volume is available.  The slices are defined as those in the native functional space.  Typically, a significance map from a particular contrast is thresholded and laid over a structural slice.  Additionally, for event-related studies the hemodynamic response for each condition can be viewed by clicking on a particular voxel in the slice.  To see the individual results for all subjects for a single slice, run sliceview-sess from the Study Directory with the following options:

-slice      slicenumber     number of the slice to view

-analysis analysisname   name of analysis (from mkanalysis-sess)

-contrast contrastname   name of contrast  (from mkcontrast-sess)

-map       mapname        name of map produced by stxgrinder-sess (sig, t, minsig, iminsig)

-sf 	sessidfile	   see above

-df	sessdirfile	   see above

The results for each subject will be displayed sequentially (ie, you must close the window for one subject before you can see the next). The T1 EPI image as the underlay.  If the T1 does not exist, it will use mean functional image as the underlay.  For event-related studies, the hemodynamic responses will  be automatically loaded.  See Appendix A for more details on how to operate the viewer.

Variation 1: Viewing raw/motion-corrected time courses.  To view a raw or motion corrected time course, cd into the Study Directory and run sliceview with the following options:

-raw  runnumber

-slice      slicenumber     number of the slice to view

-sf 	sessidfile	   see above

-df	sessdirfile	   see above

where runnumber is the number of the run whose raw data you want to view.  To view motion corrected data, add the flag "-motioncor".  If the functional subdirectory is not named "bold" then use the –fsd flag to specify the name.  The –raw and –fsd flags can be use at the same time with –analysis, -contrast, and –map flags to simultaneously view raw time courses, hemodynamic responses, and statistical maps.

Variation 2: Viewing Individual Talairach-Resampled Results. Add –space tal (cannot view raw data with tal).

21. Visualization – Individual – Mosaic-based

Note: as of 11/10/00, it is not necessary to create a mosaic before viewing a mosaic as sliceview-sess can create mosaics on-th-fly. However, if a mosaic exists, sliceview-sess will use it instead of spending time creating it. Pre-computed mosaics do use more disk space, especially for raw data. To have sliceview-sess create the mosaic on-the-fly just run it as described above with        “-slice mos”.

All of the data for a subject can be viewed at one time with sliceveiw-sess if a mosaic is created first.  A mosaic needs to be created for each data type to be viewed (ie, underlay, significance, and hemodynamic responses).  To create a mosaic of the T2 conventionals, run mkmosaic-sess from the Study Directory with the following options:

-base    t2conv

-sf 	sessidfile	   see above

-df	sessdirfile	   see above

To create a mosaic for the T1 EPIs, use t1epi as the argument for the –base flag. To create a mosaic of the subjects hemodynamic averages, run mkmosaic-sess from the Study Directory with the following options:

-analysis analysisname

-sf 	sessidfile	   see above

-df	sessdirfile	   see above

To create a mosaic of the subjects' significances for a particular contrast, run mkmosaic-sess from the Study Directory with the following options:

-contrast contrastname

-analysis analysisname

-map       mapname         name of map produced by stxgrinder-sess (sig, t, minsig, iminsig)

-sf 	sessidfile	   see above

-df	sessdirfile	   see above

To view the mosaics, run sliceview-sess as described above but use "-slice mos". To create a mosaic of the individual results resliced into talairach space, run with “-space tal”.

# 22. Visualization -– Individual--Surface-based with paint-sess and surf-sess

Surfaced-based visualization allows the user to view the results of the functional analysis on the cortical surface.  This surface can be folded, inflated, or flattened.  The visualization requires two stages: "painting" and "surfing".  "Painting" is the process of converting from native (Cartesian) space to cortical surface space.  "Surfing" is the process of viewing the results on the surface.  Note that these same programs (with different options) are used to view group-averaged results. To run each program, you must be in the Study Directory.  To paint, run

paint-sess  -analysis analysisname

–contrast contrastname

-map       mapname   (sig, t, minsig, iminsig)

–sf sessidfile –df  sessdirfile

To surf, run

surf-sess  -analysis analysisname

-contrast contrastname

-map       mapname   (sig, t, minsig, iminsig)

-hemi hemisphere                   left (lh) or right (rh) hemisphere to view

-sf sessidfile –df  sessdirfile

Customizations:  View individual data after Talariach or spherical transform (-space).  Specify a particular post-stimulus delay (-offset).

# 23. Visualization -– Group--Surface-based with paint-sess and surf-sess

Surfaced-based visualization allows the user to view the results of the functional analysis on the cortical surface.  This surface can be folded, inflated, or flattened.  The visualization requires two stages: "painting" and "surfing".  "Painting" is the process of converting from native (Cartesian) space to cortical surface space.  "Surfing" is the process of viewing the results on the surface.  Note that these same programs (with different options) are used to view individual results. To run each program, you must be in the Study Directory.  To paint, run

paint-sess  -analysis analysisname

–contrast contrastname

-map       mapname   (sig, t, minsig, iminsig)

–s groupname

-d .

-space  spacename

-isxavg method  (inter-subject averaging method: fixed or random)

-subject subjectname  (for group data, need to specify an individual subject

on whose surface the data will be rendered).

Note: as of 11/10/00, paint-sess will paint all frames in the map.

To surf, run

surf-sess  -analysis analysisname

-contrast contrastname

-map       mapname   (sig, t, minsig, iminsig)

-offset framenumber   (zero-based frame number in map)

-hemi hemisphere                   left (lh) or right (rh) hemisphere to view

–s groupname

-d .

-space  spacename

-isxavg method  (inter-subject averaging method: fixed or random)

-subject subjectname  (same as in paint-sess)

Offset/FrameNumber: for event-related designs, the sig map will have multiple significance values for each voxel corresponding to each post-stimulus delay in the time window. When surfing, the user must specify which post-stimulus delay to be viewed with the “-offset” flag. The value of the offset corresponds to the nth post-stimulus delay with n=0 being the first. For example, if the time window (from mkanalysis-sess) is 20 seconds with a pre-stimulus window of 4 seconds and a TR of 2, then there will be 10 post-stimulus delays points in the window, two of which will actually be before the stimulus onset. Thus, the 10 points in the window correspond to –4, -2, 0, +2, +4, +6, +8, +10, +12, and +14 post-stimulus delay seconds. The map 4 seconds after stimulus onset corresponds to the 5th point, so the offset would be 4. Note for blocked designs or for viewing the minsig map, the offset is automatically set to 0.

24. Visualization – Volume-based.

To view results for an individual (ie, hemodynamic responses and statistical maps) overlaid on the 3D anatomical in the Anatomical Database, cd to the Study Directory and run tkmedit-sesss with the following options:

-contrast contrastname

-analysis analysisname

-map       mapname         name of map produced by stxgrinder-sess (sig, t, minsig, iminsig)

-s 	sessid		   see above

-df	sessdirfile	   see above

This will bring up several windows:  an image window, a tools window, and, for event-related analyses, a hemodynamic response window.  The image window will display a coronal slice from the Anatomical Database for that subject with activation overlay.  The user can navigate through the volume by changing the cardinal view in the Tools window by clicking on “Sagittal”, “Horizontal”, or “Coronal”.  The slice within that cardinal view can be manipulated with the slider underneath the button for that view.  The user can also zoom in and out using the Zoom slider.  All three cardinal axes can be viewed simultaneously by activating the “Display” pull-down menu and selecting “3D Multiple Views”.  Clicking on a point in the image window will move the cursor to that point, change the other two views to intersect with the this point, and show the hemodynamic response for all conditions at that point.  The current time-point can be changed by activating the Configure Functional Overlay window from the “Display” pull-down menu.  Adjust the “Time Point” value and hit “Apply”.  The hemodynamic response window can be manipulated by activating the Configure Time Course window, also from the Display pull-down.  To quit from tkmedit, activate the “File” pull-down menu and select Quit.

Variation 1: Viewing group talairach results.  Call tkmedit-sess with the above arguments but with  “ –s groupname –space tal –isxavg method” where “method” is either fixed or random depending upon from which intersubject averaging method you want to view the results.

25. Random-effects Inter-subject Averaging using isxavg-re-sess

Once the subjects have been resampled into a common space (Talairach or spherical), the averages from the individual subjects can be averaged together using a Random-Effects model (see Step 18 for Fixed Effects averaging).  You may want to combine all of your subjects into a grand average, or you may want to divide your subjects into groups and compute grand averages for each group.  In any event, you will need to specify which subjects will be included in the average.  This collection of subjects will be referred to as a group, and you will be required to assign each group a name (groupname) which will be needed for subsequent processing.  Unlike Fixed-Effect averaging, you must also supply a contrast to test. Within the Study Directory, run isxavg-re-sess with the following options:

-analysis analysisname	name of analysis to resample

-contrast contrastname            name of contrast to test

-group groupname                  name of collection of subjects

-space  spacename                  name of the resampling space (tal or sph)

-sf 	sessidfile		see above

-df	sessdirfile		see above

This will create a directory called groupname in the Study directory.  This directory will have the same structure as an individual session and can be accessed with the –s and –sf  flags as if it were an individual session.  Inside this directory, you will see bold/analysisname/spacename-rfx in which there will be one volume, h-offset, in which the average EPI image is stored. Under this directory, there will be another directory called “contrastname” in which the results of the random-effects averaging are stored. Within this directory, five new volumes will be created: avg, t, sig, minsig, iminsig. The avg volume holds the averages and standard deviations that results from the application of the contrast and inter-subject averaging.  The other volumes are interpreted the same as in Step 12).

### APPENIX A: SLICEVIEW OPERATION

Sliceview is one way of visualizing the statistical maps generated by the Standard Stream.  It can display maps laid over T1 or T2 structural images (slices or mosaics of slices) and can also display the average hemodynamic changes in response to different event types. See steps 20 and 21 for how to invoke sliceview and create mosaics.  Sliceview is actually a front-end for a program called yakview documentation for which is in ~inverse/fmri/docs/yakview.ps. When sliceview is run, it immediately brings up an image window displaying the structural and overlay slices.  There is a color scale that indicates how color maps to value.  For cases in which the overlay is a significance map, then the values will be the log10 of the significance values.  The minimum and maximum thresholds are set when invoking slices view by using the –thmin and –thmax options or by hitting ‘o’ inside the image window. Clicking with the mouse on a voxel will cause the voxel location and overlay value to be printed above the image.  The voxel selected by the mouse is referred to as the current voxel. The various functions of sliceview are controlled by hitting keys when the sliceview window is open.  Hitting 'h' brings up a menu of control keys.

Image Status Bar: the status bar is just above the image (this may not be visible during zoom).  The first set of numbers gives the index of the current voxel in the structural image.  The second set gives the index in the map.  The third number is the value of the map at the current voxel.  On the right side of the image, the fields show the current the current plane, sign of the tail, and the range of values for the color scale.

Image Control Keys: Initially both the positive (red/yellow) and negative (blue) tails will be displayed.  Hitting 't' will cause only the positive tails to be displayed.  Hitting 't' again will cause only the negative tails to be displayed.  Hitting 't' again brings it back to positive/negative.  For maps with multiple planes (as in the "sig" map for event-related), hitting '+' and '-' scrolls through the different planes.  Press 'r' to bring up another window that shows the time course of the raw fMRI signal at the current voxel.  Press 'g' to bring up a window that displays the hemodynamic responses (event-related only).  Hitting ‘o’ will bring up a window that allows the user set the thresholds for the overlay color scale.

Viewing the Hemodynamic Responses: the hemodynamic response (HDR) window is accessed by pressing 'g' in the image window.  As in the image window, the HDR window is modified by a series of control keys.  To see a list of control keys, activate the HDR window and press 'h'.  The HDR window shows the average response to each event type at the current voxel.  The horizontal axis is time (in seconds) sampled at the TER.  The vertical axis is in raw fMRI units (unless the percent signal change option is used).

HDR Status Bar:  there are a series of text fields just above the HDR plot that indicate the current status of the window.  The first field is a series of numbers (and possibly asterices) that indicate which conditions are currently being shown.  The next field is either 'YFree' or 'YHold'. 'YFree' indicates that the y-axis will scale with the data.  The field is either 'BLFree' or 'BLZero' indicating whether the prestimulus baseline has been zeroed.  The next two fields indicate the index of the current voxel.  The next field with be either blank or '%' to indicate whether percent baseline is being viewed.  Finally, the last field gives the baseline (or offset) value at the current voxel.

HDR Control Keys: pressing 'h' when the HDR window is active will bring up a help menu with the available control keys and a short description of what they do.  Pressing a number key will toggle the display of that condition (unviewed conditions have an asterix in their status field).  Pressing 'e' will toggle the display of standard error bars.  Pressing 'p' will toggle display of percent signal change.  Pressing 'y' will toggle whether the y-axis will change with the data (YFree) or stay fixed at the current range (YHold).  Press 'v' to save the plot values to an ascii file.

### APPENDIX B: EVENT-RELATED ANALYSIS OF BLOCKED DESIGNS

Often, experimenters want to view the average time-point-by-time-point response to a block. However, as described so far, the Standard Stream only allows for analysis of blocked design where all the time-points in a block are averaged together.  This appendix describes how to obtain the response to the entire block.  First, create new paradigm files.  Instead of representing the block by a separate entry for each time-point in the block, put only one entry corresponding to the onset of the block.  Next, run mkanalysis-sess indicating the name of the new paradigm file and giving a new analysisname. Do not use the –blocked flag. Edit the file called 'analysis.cfg' in the analysisname subdirectory (see Step 9 above).  Change the 'timewindow' field to be twice the width of the block (in seconds) minus 1 TR.  Change the 'prestim' window to be half the width of the block (in seconds).  Continue as if it were any other event-related design.  The hemodynamic response to a block event will now be the average of all the blocks of that type starting half a block before the onset of the block and extending to half a block after the offset of the block.

### APPENDIX C: FIXSEQINFO-SESS

As of 11/10/00, many of the programs in the standard stream will be expecting that the seq.info file to have a new format. This file is located in the directory directly above the subdirectory where the data for a run is stored (eg, in the bold or t2conv directories). The seq.info file will already be in this format for data sets unpacked after 11/10/00. For data sets unpacked before that date, the user must run a new program called fixeseqinfo-sess.  To do this, cd to the Study Directory, and type:

fixseqinfo-sess

[session identification options   –s/-sf/-d/-df]

-fsd    functional-subdirectory (if other than “bold”)

-inplaneres   in-plane resolution (in mm)

-betplaneres  between-plane resolution (in mm)

-TR TR in seconds (for functional scans)

The in-plane resolution is the size of the pixels within a slice. The between-plane resolution is the distance between slices (note: this is not the same as the slice thickness if a slice “skip” has been used).  Remember, it is often the case that the in-plane resolution a bold sequence is different that of the t2conv sequence.

For example:

fixseqinfo-sess –fsd bold2–inplaneres 3.125  –betplaneres 5.0  –TR 2 –sf sessid –df sessdir

would fix the seq.info file in the bold2 subdirectory for those sessions indicated by the –sf/-df argumens.

Note: this program needs to be run for each sequence type collected in the session (eg, bold, t1epi, t2conv, etc) excluding anatomical runs (eg, 3danat).

### REFERENCES

Burock, M. A. (1998). Design and statistical analysis of fMRI experiments to assess human brain hemodynamic responses. Unpublished Master’s Thesis, M.I.T.

Burock, M. A., Buckner, R. L., Woldorff, M. G., Rosen, B. R., & Dale, A. M. (1998). Randomized event-related experimental designs allow for extremely rapid presentation rates using functional MRI.  Neuroreport,9, 37 35-3739.

Dale, A.M., Fischl, Bruce, Sereno, M.I., (1999). Cortical Surface-Based Analysis I: Segmentation and Surface Reconstruction. NeuroImage 9(2):179-194.

Dale, A.M., Greve, D.N., and Burock, M.A. (1999) Optimal Stimulus Sequences for Event-Related fMRI.  Human Brain Mapping.

Fischl, Bruce, Sereno, M.I., Dale, A.M., (1999). Cortical Surface-Based Analysis II: Inflation, Flattening, and a Surface-Based Coordinate System. NeuroImage 9(2):195-207.

Fischl, Bruce, Dale, A.M., Sereno, M.I., Tootell, R.B.H. and Rosen, B.R. (1998) A Coordinate System for the Cortical Surface. NeuroImage 7(4):S740

Talairach, J., & Tournoux, P. (1988). Co-planar stereotaxic atlas of the human brain. New York: Times Medical Publishing.

As of the time of this writing, the standard stream does not support inter-group analysis.

Note that this may be different than the first image acquired if some of the images at the beginning of the run are discarded  (ie, DDAs).

There is no post-stimulus window parameter.  Rather, the entire time window and the pre-stimulus window are specified.  The post-stimulus window equals the entire time window minus the pre-stimulus window.

Note to xds users: xds sets the activation color scale assuming that the significance values  have been transformed by the natural log.

Currently, there is a problem with the unpacking of the T1 EPI which prevent viewing.

There is currently a bug in sliceview which causes to color scale to come up with the wrong range.  Hitting '+' followed by '-' will restore the correct range.